Monthly Quality Update

July 2015

OHIO RIVER VALLEY

WATER SANITATION COMMISSION

5735 Kellogg Avenue, Cincinnati, Ohio  45230	(513) 231-7719	Fax (513) 231-7761

## Flows

Average flows at seven Ohio River locations are displayed below.  At each location the July 2015 average flow (black bar) is contrasted with the long-term average monthly flow (gray bar).

## Bacteria

The bacteria monitoring results from six Ohio River locations in July 2014 are in the attached graphs. Beginning with the 2015 Contact Recreation season (April through October), there are thirteen sites being sampled.  ORSANCO’s water quality criteria for the protection of human health during the contact recreation season are:

         E. coli count is not to exceed 240 CFU/100mL in more than 25 percent of samples.           (E.coli is shown with a red line on the graph.)

         E. coli shall not exceed 130 CFU/100mL as a 90-day geometric mean, based on                not less than five samples per month.

ORSANCO’s water quality criteria for the protection of public water supply year round are:

        Fecal Coliform shall not exceed 2,000/100mL as a monthly geometric mean based            on not less than five samples per month.

#### Pittsburgh

In Pittsburgh, ORM 1.4 and ORM 4.3 both had two individual exceedances for E.coli.

#### Wheeling

In Wheeling, ORM 86.8 did not exceed the E.coli criteria. ORM 92.8 had one individual exceedance for E.coli.

Huntington

In Huntington, ORM 305.1 and ORM 314.8 both had two individual exceedances for E.coli.

Cincinnati

In Cincinnati, ORM 462.6, ORM 470.0 and ORM 477.5 all had two individual exceedances for E.coli.

#### Louisville

In Louisville, ORM 594.0 had one individual exceedance for E.coli. ORM 619.3 had two individual exceedances for E.coli. Due to unfavorable sampling conditions at ORM 619.3 only four samples were taken this month.

Evansville

In Evansville, ORM 791.5 did not exceed the E.coli criteria. ORM 793.7 had two individual exceedances for E.coli.

Dissolved Oxygen and Temperature

A summary of the dissolved oxygen and temperature readings collected in July 2015 are displayed in the attached tables.  During the months of May through October, ORSANCO interrogates six stations for dissolved oxygen (D.O.) and temperature data once per day, Monday through Friday, for the previous 24-hour period.  This data is compiled in-house to produce a report showing the minimum, maximum and average of the acquired D.O. and temperature readings.  This year Olmstead was added and the results are instantaneous data. ORSANCO’s water quality criteria for the protection of warm water aquatic life habitat are:

- The minimum D.O. concentration shall not be less than 4.0 mg/L at any time.

- Average D.O. concentration shall be at least 5.0 mg/L for each calendar day.

- During the April 15 – June 15 spawning season, a minimum D.O. concentration of 5.0 mg/L shall be maintained at all times.

- The monthly average of the daily maximum for July 1-31 is:

  - 89.0 °F at MP 0 to MP 341.

  - 89.0 °F at MP 341.1 to MP 606.8.

  - 89.0 °F at MP 606.9 to MP 981.

Greenup, McAlpine and Olmstead all had instrument issues during the month of July.  There were no temperature violations due to the cool temperatures we have been experiencing this summer.

Please click on this link to be directed to the DO/Temperature tables for July 2015: http://www.orsanco.org/dissolved-oxygen-and-temperature

Atrazine

Evansville drinking water utility samples the Ohio River at their intake and analyzes for atrazine by immunoassay technique. Results for July 2015 are presented below. Evansville had no criteria violations of the Maximum Contaminant Level (MCL) of 3.0 ug/L set by the Safe Drinking Water Act for finished drinking water.