Monthly Quality Update

June 2016

OHIO RIVER VALLEY

WATER SANITATION COMMISSION

5735 Kellogg Avenue, Cincinnati, Ohio  45230	(513) 231-7719	Fax (513) 231-7761

## Flows

Average flows at seven Ohio River locations are displayed below.  At each location the June 2016 average flow (black bar) is contrasted with the long-term average monthly flow (gray bar).  At the upper and lower locations the June 2016 average flow were below the long-term average monthly flow. At the two middle locations the June 2016 average flow were slightly above the long-term average monthly flow.

## Bacteria

The bacteria monitoring results from six Ohio River locations in June 2016 are in the attached graphs. Beginning with the 2016 Contact Recreation season (April through October), there are thirteen sites being sampled.  ORSANCO’s water quality criteria for the protection of human health during the contact recreation season are:

         E. coli count is not to exceed 240 CFU/100mL in more than 25 percent of samples.           (E.coli is shown with a red line on the graph.)

         E. coli shall not exceed 130 CFU/100mL as a 90-day geometric mean, based on                not less than five samples per month.

ORSANCO’s water quality criteria for the protection of public water supply year round are:

        Fecal Coliform shall not exceed 2,000/100mL as a monthly geometric mean based            on not less than five samples per month.

#### Pittsburgh

In Pittsburgh, ORM 1.4M and ORM 4.3 had two individual exceedances for E.coli.

#### Wheeling

In Wheeling, ORM 86.8 did not exceed the E.coli criteria. ORM 92.8 had one individual exceedance for E.coli.

Huntington

In Huntington, ORM 305.1 and ORM 314.8 had two individual exceedances for E.coli.

Cincinnati

In Cincinnati, ORM 462.6 and ORM 470.0 had one individual exceedance for E.coli.

ORM 477.5 had two individual exceedances for E.coli.

#### Louisville

In Louisville, ORM 594.0 and ORM 619.3 had one individual exceedance for E.coli.

Evansville

In Evansville, ORM 791.5 had one individual exceedance for E.coli.  ORM 793.7 had two individual exceedances for E.coli.

Dissolved Oxygen and Temperature

During the months of June through October, ORSANCO interrogates nine stations for dissolved oxygen (D.O.) and temperature data once per day, Monday through Friday, for the previous 24-hour period.  This data is compiled in-house to produce a report showing the minimum, maximum and average of the acquired D.O. and temperature readings. ORSANCO’s water quality criteria for the protection of warm water aquatic life habitat are:

- The minimum D.O. concentration shall not be less than 4.0 mg/L at any time.

- Average D.O. concentration shall be at least 5.0 mg/L for each calendar day.

- During the April 15 – June 15 spawning season, a minimum D.O. concentration of 5.0 mg/L shall be maintained at all times.

- The monthly average of the daily maximum for June 1-15 is:

    - 78.8 °F at MP 0 to MP 341.

    - 83.5 °F at MP 341.1 to MP 606.8.

    - 77.6 °F at MP 606.9 to MP 981.

- The monthly average of the daily maximum for June 16-30 is:

    - 87.0 °F at MP 0 to MP 341.

    - 87.0 °F at MP 341.1 to MP 606.8

    - 87.0 °F at MP 606.9 to MP 981.

Racine, Greenup, Markland, and McAlpine saw daily DO averages fall below 5.0 mg/L. There were no temperature exceedances for the month of June.

Please click on this link to be directed to the DO/Temperature tables for June 2016: http://www.orsanco.org/dissolved-oxygen-and-temperature

Atrazine

Evansville drinking water utility samples the Ohio River at their intake and analyzes for atrazine by immunoassay technique. Results for June 2016 are presented below. Evansville had no criteria violations of the Maximum Contaminant Level (MCL) of 3.0 ug/L set by the Safe Drinking Water Act for finished drinking water.