Monthly Quality Update

September 2017

OHIO RIVER VALLEY

WATER SANITATION COMMISSION

5735 Kellogg Avenue, Cincinnati, Ohio  45230	(513) 231-7719	Fax (513) 231-7761

## Flows

Average flows at seven Ohio River locations are displayed below.  At each location the September 2017 average flow (black bar) is contrasted with the long-term average monthly flow (gray bar).

## Bacteria

The bacteria monitoring results from six Ohio River locations in September 2017 are in the attached graphs. There are thirteen sites being sampled and West Virginia is the only state that has both Fecal Coliform and E.coli results.  ORSANCO’s water quality criteria for the protection of human health during the contact recreation season are:

         E. coli count is not to exceed 240 CFU/100mL in more than 25 percent of samples.           (E.coli is shown with a red line on the graph.)

         E. coli shall not exceed 130 CFU/100mL as a 90-day geometric mean, based on                not less than five samples per month.

ORSANCO’s water quality criteria for the protection of public water supply year round are:

        Fecal Coliform shall not exceed 2,000/100mL as a monthly geometric mean based            on not less than five samples per month.

#### Pittsburgh

In Pittsburgh, ORM 1.4M and ORM 4.3 did not exceed the E.coli criteria. The sample at ORM 1.4M on September 21 had an equipment issue and was not analyzed.

#### Wheeling

In Wheeling, ORM 86.8 did not exceed the E.coli criteria. ORM 92.8 had one individual exceedance for E.coli.

Huntington

In Huntington, ORM 305.1 did not exceed the E.coli criteria. ORM 314.8 had one individual exceedance for E.coli.

Cincinnati

In Cincinnati, ORM 462.6, ORM 470.0 and ORM 477.5 did not exceed the E.coli criteria.

#### Louisville

In Louisville, ORM 594.0 and ORM 619.3 did not exceed the E.coli criteria.

Evansville

In Evansville, ORM 791.5 did not exceed the E.coli criteria. ORM 793.7 had two individual exceedances for E.coli.

Dissolved Oxygen and Temperature

A summary of the dissolved oxygen and temperature readings collected in September 2017 are displayed in the attached tables.  During the months of May through October, ORSANCO interrogates eleven stations for dissolved oxygen (D.O.) and temperature data once per day, Monday through Friday, for the previous 24-hour period.  This data is compiled in-house to produce a report showing the minimum, maximum and average of the acquired D.O. and temperature readings.  This year Olmstead was added and the results are instantaneous data. ORSANCO’s water quality criteria for the protection of warm water aquatic life habitat are:

- The minimum D.O. concentration shall not be less than 4.0 mg/L at any time.

- Average D.O. concentration shall be at least 5.0 mg/L for each calendar day.

- During the April 15 – June 15 spawning season, a minimum D.O. concentration of 5.0 mg/L shall be maintained at all times.

- The monthly average of the daily maximum for September 1-15 is:

  - 87.0 °F at MP 0 to MP 341.

  - 87.0 °F at MP 341.1 to MP 606.8.

  - 87.0 °F at MP 606.9 to MP 981

- The monthly average of the daily maximum for September 16-30 is:

  - 81.0 °F at MP 0 to MP 341.

  - 84.7 °F at MP 341.1 to MP 606.8.

  - 82.6 °F at MP 606.9 to MP 981.

Kyger and Markland saw D.O. levels drop below the average of 5.0 mg/L. There were no temperature violations.

Please click on the link below to be directed to the DO/Temperature tables for August 2017:

http://www.orsanco.org/dissolved-oxygen-and-temperature

Atrazine

Evansville drinking water utility samples the Ohio River at their intake and analyzes for atrazine by immunoassay technique. Results for September 2017 are presented below. Evansville had no criteria violations of the Maximum Contaminant Level (MCL) of 3.0 ug/L set by the Safe Drinking Water Act for finished drinking water.