| # Memo | Ohio River Valley Water Sanitation Commission 5735 Kellogg Avenue Cincinnati, Ohio 45230 Telephone: (513) 231-7719 Fax: (513) 231-7761 |
| --- | --- |

DATE:	August 27, 2012

### TO:	Technical Committee

Toby Frevert, Chairman	Allen J. Newman, VA	Eriks Janelsins, PIACO

Marcia Willhite	, IL		   Patrick Campbell, WV	      MaryLynn Lodor, POTW

Thomas Easterly, IN	Deborah Lee, USCOE	Jack Wang, WUAC

Randy Payne, KY	William R. Guertal, USGS	Rob Reash, PIAC

Gerard Palumbo, NY	John Armstead, U.S. EPA

George Elmaraghy, OH	Peter Tennant

### Ronald Schwartz, PA

FROM:	Stacey Cochran

SUBJECT:	Quality Update –July 2012 Conditions

## Flows

Average flows at seven Ohio River locations are displayed below.  At each location the July 2012 average flow (black bar) is contrasted with the long-term average monthly flow (gray bar).  The flows were well below the long-term average for the month of July.

## Bacteria

The bacteria monitoring results from six Ohio River locations in July 2012 are in the attached graphs. Beginning with the 2012 Contact Recreation season (May through October), there are thirteen sites being sampled.  ORSANCO’s water quality criteria for the protection of human health during the contact recreation season are:

         Fecal coliform count is not to exceed 400 CFU/100mL in any single sample. (Fecal            coliform is shown with a blue line on the graph.)

         Fecal coliform monthly geometric mean of at least five samples is not to exceed                200 CFU/100mL.

         E. coli count is not to exceed 240 CFU/100mL in any single sample. (E.coli is                   shown with a red line on the graph.)

         E. coli monthly geometric mean of at least five samples is not to exceed 130                     CFU/100mL.

#### All Sites

The site from each location with the highest monthly geometric mean for fecal coliform and E.coli are plotted in the first graph of the attached bacteria monitoring results.

#### Pittsburgh

In Pittsburgh, ORM 1.4M had two individual exceedances for fecal coliform and no exceedances for E.coli. ORM 4.3 had three individual exceedances for fecal coliform and two individual exceedances for E.coli. The monthly geometric mean for fecal coliform was exceeded at both sites. The monthly geometric mean for E.coli was exceeded at ORM 4.3 only.

#### Wheeling

In Wheeling, ORM 86.8 had no exceedances for fecal coliform or E.coli. ORM 92.8 had one individual exceedance for fecal coliform and two individual exceedances for E.coli.  The monthly geometric mean for fecal coliform was exceeded at ORM 92.8 and the monthly geometric mean for E.coli was not exceeded.

#### Huntington

In Huntington, ORM 305.1 had no exceedances for fecal coliform or E.coli. ORM 314.8 had one individual exceedance for fecal coliform and no exceedances for E.coli. The monthly geometric mean for both fecal coliform and E.coli was not exceeded.

#### Cincinnati

In Cincinnati, ORM 462.6 and ORM 470.0 had no exceedances for fecal coliform or E.coli.  ORM 477.5 had two individual exceedances for both fecal coliform and E.coli. The monthly geometric mean for both fecal coliform and E.coli was not exceeded.

#### Louisville

In Louisville, ORM 594.0 had no exceedances for fecal coliform or E.coli. ORM 619.3 had three individual exceedances for fecal coliform and two individual exceedances for E.coli. The monthly geometric mean for both fecal coliform and E.coli was exceeded at ORM 619.3.

#### Evansville

In Evansville, ORM 791.5 had no exceedances for fecal coliform or E.coli. ORM 793.7 had three individual exceedances for fecal coliform and two individual exceedances for E.coli. The monthly geometric mean for fecal coliform was exceeded at ORM 793.7.

Dissolved Oxygen and Temperature

A summary of the dissolved oxygen and temperature readings collected in July 2012 are displayed in the attached tables.  During the months of May through October, ORSANCO interrogates nine stations for dissolved oxygen (D.O.) and temperature data once per day, Monday through Friday, for the previous 24-hour period.  This data is compiled in-house to produce a report showing the minimum, maximum and average of the acquired D.O. and temperature readings.  ORSANCO’s water quality criteria for the protection of warm water aquatic life habitat are:

- The minimum D.O. concentration shall not be less than 4.0 mg/L at any time.

- Average D.O. concentration shall be at least 5.0 mg/L for each calendar day.

- During the April 15 – June 15 spawning season, a minimum D.O. concentration of 5.0 mg/L shall be maintained at all times.

- The temperature period average for July 1-31 is 84°F and the maximum is 89°F.

The instruments at Racine and Markland had some calibration issues and were being corrected. This year due to the low flow and heat, some stations are seeing an increase of zebra mussels on the instruments and this is causing an issue. Hannibal, Greenup (downstream), Markland, and McAlpine all exceeded the Period Average for temperature during the month of July.