| # Memo | Ohio River Valley Water Sanitation Commission 5735 Kellogg Avenue Cincinnati, Ohio 45230 Telephone: (513) 231-7719 Fax: (513) 231-7761 |
| --- | --- |

DATE:	July 17, 2013

### TO:		Technical Committee

Stuart Bruny, Chairman	Erich Emery, USACE

Marcia Willhite, IL		Thomas Kaminski, USCG

Shivi Selvaratnam, IN		Ed Ambrogio, USEPA

Randy Payne, KY		William Guertal, USGS

Jeff Konsella, NY		Ex Officio1

George Elmaraghy, OH	Rob Reash, Chairman, PIAC

Ronald Schwartz, PA		Eriks Janelsins, Chairman, PIACO

Melanie Davenport, VA	MaryLynn Lodor, Chairman, POTW

Scott Mandirola, WV		Ron Bargiel, Chairman, WUAC

Peter Tennant, Chief Engineer

FROM:	Stacey Cochran

SUBJECT:	Quality Update –June 2013 Conditions

## Flows

Average flows at seven Ohio River locations are displayed below.  At each location the June 2013 average flow (black bar) is contrasted with the long-term average monthly flow (gray bar).

## Bacteria

The bacteria monitoring results from six Ohio River locations in June 2013 are in the attached graphs. Beginning with the 2013 Contact Recreation season (April through October), there are thirteen sites being sampled.  ORSANCO’s water quality criteria for the protection of human health during the contact recreation season are:

         E. coli count is not to exceed 240 CFU/100mL in more than 25 percent of samples.           (E.coli is shown with a red line on the graph.)

         E. coli shall not exceed 130 CFU/100mL as a 90-day geometric mean, based on                not less than five samples per month.

ORSANCO’s water quality criteria for the protection of public water supply year round are:

- Fecal Coliform shall not exceed 2,000/100mL as a monthly geometric mean based on not less than five samples per month.

#### Pittsburgh

In Pittsburgh, ORM 1.4M had one individual exceedance for E.coli. ORM 4.3 had two individual exceedances for E.coli.  The 90 day geometric mean for E.coli was not exceeded at either site.

#### Wheeling

In Wheeling, ORM 86.8 did not exceed the E.coli criteria.  ORM 92.8 had three individual exceedances for E.coli. The 90 day geometric mean for E.coli was exceeded at ORM 92.8.

#### Huntington

In Huntington, ORM 305.1 did not exceed the E.coli criteria. ORM 314.8 had one individual exceedance for E.coli. The 90 day geometric mean for E.coli was not exceeded at either site.

#### Cincinnati

In Cincinnati, ORM 462.6, ORM 470.0 and ORM 477 did not exceed the E.coli criteria. The 90 day geometric mean for E.coli was not exceeded at any site.

#### Louisville

In Louisville, ORM 594.0 had one individual exceedance for E.coli.  ORM 619.3 did not exceed the E.coli criteria. The 90 day geometric mean for E.coli was not exceeded at either site.

#### Evansville

In Evansville, ORM 791.5 had one individual exceedance for E.coli. ORM 793.7 had two individual exceedances for E.coli.  The 90 day geometric mean for E.coli was exceeded at ORM 793.7.

Dissolved Oxygen and Temperature

A summary of the dissolved oxygen and temperature readings collected in June 2013 are displayed in the attached tables.  During the months of May through October, ORSANCO interrogates nine stations for dissolved oxygen (D.O.) and temperature data once per day, Monday through Friday, for the previous 24-hour period.  This data is compiled in-house to produce a report showing the minimum, maximum and average of the acquired D.O. and temperature readings.  ORSANCO’s water quality criteria for the protection of warm water aquatic life habitat are:

- The minimum D.O. concentration shall not be less than 4.0 mg/L at any time.

- Average D.O. concentration shall be at least 5.0 mg/L for each calendar day.

- During the April 15 – June 15 spawning season, a minimum D.O. concentration of 5.0 mg/L shall be maintained at all times.

- The temperature period average for June 1-15 is 80°F and the maximum is 85°F.

- The temperature period average for June 16-30 is 83°F and the maximum is 87°F.

Kyger was the only station with D.O. violations for the month of June. Both Hannibal and Greenup had instrument issues and are being addressed. There were no Temperature violations for the month of June.

Daily D.O. Readings for June 2013

Daily Temperature Readings for June 2013

*These tables are also found on our website under the data section.