| # Memo | Ohio River Valley Water Sanitation Commission 5735 Kellogg Avenue Cincinnati, Ohio 45230 Telephone: (513) 231-7719 Fax: (513) 231-7761 |
| --- | --- |

DATE:	November 13, 2013

### TO:		Technical Committee

Stuart Bruny, Chairman	Erich Emery, USACE

Marcia Willhite, IL		Thomas Kaminski, USCG

Shivi Selvaratnam, IN		Ed Ambrogio, USEPA

Randy Payne, KY		William Guertal, USGS

Jeff Konsella, NY		Ex Officio1

Paul Novak/Scott Nally, OH	Rob Reash, Chairman, PIAC

Ronald Schwartz, PA		Eriks Janelsins, Chairman, PIACO

Melanie Davenport, VA	MaryLynn Lodor, Chairman, POTW

Scott Mandirola, WV		Ron Bargiel, Chairman, WUAC

Peter Tennant, Chief Engineer

FROM:	Stacey Cochran

SUBJECT:	Quality Update –October 2013 Conditions

## Flows

Average flows at seven Ohio River locations are displayed below.  At each location the October 2013 average flow (black bar) is contrasted with the long-term average monthly flow (gray bar).  The average flow for October 2013 is below the long-term monthly flow at every location.

## Bacteria

The bacteria monitoring results from six Ohio River locations in October 2013 are in the attached graphs. Beginning with the 2013 Contact Recreation season (April through October), there are thirteen sites being sampled.  ORSANCO’s water quality criteria for the protection of human health during the contact recreation season are:

         E. coli count is not to exceed 240 CFU/100mL in more than 25 percent of samples.           (E.coli is shown with a red line on the graph.)

         E. coli shall not exceed 130 CFU/100mL as a 90-day geometric mean, based on                not less than five samples per month.

ORSANCO’s water quality criteria for the protection of public water supply year round are:

- Fecal Coliform shall not exceed 2,000/100mL as a monthly geometric mean based on not less than five samples per month.

#### Pittsburgh

In Pittsburgh, ORM 1.4M and ORM 4.3 both had one individual exceedance for E.coli. The 90 day geometric mean could not be calculated due to partial data collected for August and September.

#### Wheeling

In Wheeling, ORM 86.8 did not exceed the E.coli criteria. ORM 92.8 has one individual exceedance for E.coli. The 90 day geometric mean for E.coli was not exceeded at either site.

#### Huntington

In Huntington, ORM 305.1 and ORM 314.8 had no exceedances for E.coli. The 90 day geometric mean for E.coli was not exceeded at either site.

#### Cincinnati

In Cincinnati, ORM 462.6, ORM 470.0 and ORM 477.5 did not exceed the E.coli criteria. The 90 day geometric mean for E.coli was not exceeded at any site.

#### Louisville

In Louisville, ORM 594.0 has one individual exceedance for E.coli. ORM 619.3 had three individual exceedances for E.coli. The 90 day geometric mean for E.coli was not exceeded at either site.

#### Evansville

In Evansville, ORM 791.5 did not exceed the E.coli criteria. ORM 793.7 has one individual exceedance for E.coli. The 90 day geometric mean for E.coli was not exceeded at either site.

Dissolved Oxygen and Temperature

A summary of the dissolved oxygen and temperature readings collected in October 2013 are displayed in the attached tables.  During the months of May through October, ORSANCO interrogates nine stations for dissolved oxygen (D.O.) and temperature data once per day, Monday through Friday, for the previous 24-hour period.  This data is compiled in-house to produce a report showing the minimum, maximum and average of the acquired D.O. and temperature readings.  ORSANCO’s water quality criteria for the protection of warm water aquatic life habitat are:

- The minimum D.O. concentration shall not be less than 4.0 mg/L at any time.

- Average D.O. concentration shall be at least 5.0 mg/L for each calendar day.

- During the April 15 – June 15 spawning season, a minimum D.O. concentration of 5.0 mg/L shall be maintained at all times.

- The monthly averages of the daily maximum allowable temperature for October  at Mile Point (MP) 0-341 is MP 74.1°F, MP 341.1-MP 606.8 76.7°F, and MP 606.9-MP 981 is 75.5°F.

Racine and Kyger Station did have D.O. criteria violations for the month of October. There were no exceedances of the monthly averages of the daily maximum allowable temperature violations for the month of October.

Daily D.O. Readings for October 2013

Daily Temperature Readings for October 2013

*These tables are also found on our website under the data section.